NEXARA

Integrated Management System

Fire Risk Assessment Procedure

FSO 2005 Article 9 | PAS 79-1:2020 | Building Safety Act 2022

| Document Number: | FEM-PRO-001 |
| --- | --- |
| Version: | 1.0 |
| Date: | [DD/MM/YYYY] |
| Owner: | [Health & Safety Manager] |
| Approved By: | [Managing Director] |
| Classification: | Confidential |

Revision History

| Version | Date | Description | Author | Approved By |
| --- | --- | --- | --- | --- |
| 1.0 | [DD/MM/YYYY] | Initial Issue | [Author Name] | [Approver Name] |

# 1. Purpose

This procedure defines the process for conducting, recording, and reviewing Fire Risk Assessments (FRAs) in compliance with the Regulatory Reform (Fire Safety) Order 2005 Article 9 and the Building Safety Act 2022 Section 156.

# 2. Scope

Applies to all non-domestic premises owned, leased, or managed by [Company Name].

# 3. Legal Basis

- FSO 2005 Article 9 — Duty to carry out risk assessment

- FSO 2005 Article 11 — Duty to record significant findings for 5+ employees

- Building Safety Act 2022 s.156 — Full written record of assessment mandatory (from October 2023)

- PAS 79-1:2020 — Fire risk assessment guidance

# 4. Definitions

- Responsible Person (RP): Person with control of premises (employer, owner, occupier)

- Competent Person: Assessor with adequate training, knowledge, experience (BAFE/IFE/IFSM)

- Material Change: Any change that may affect fire risk (layout, use, occupancy, process)

# 5. Responsibilities

## 5.1 Responsible Person

Ensure a suitable and sufficient FRA is carried out by a competent person. Ensure findings are acted upon. Ensure records are maintained and shared with incoming RP on change of ownership/management.

## 5.2 Fire Risk Assessor

Conduct the assessment using the FSO 5-step methodology. Document all findings. Provide prioritised action plan.

# 6. Procedure — FSO 5-Step Fire Risk Assessment

## 6.1 Step 1: Identify Fire Hazards

Systematically identify:

Ignition Sources: Electrical equipment, hot work, smoking, arson potential, cooking, heating, machinery, static electricity, naked flames, lightning.

Fuel Sources: Paper/cardboard, wood, flammable liquids, textiles, plastics, rubber, waste materials, furniture, packaging.

Oxygen Sources: Natural air, air conditioning systems, oxygen cylinders, oxidising chemicals, ventilation systems.

For each hazard identified, record location, quantity, and proximity to ignition/fuel/oxygen.

## 6.2 Step 2: Identify People at Risk

Record numbers of: employees, visitors, contractors, members of the public who may be present. Identify persons who may be especially at risk: persons with mobility impairment (require PEEP), persons with visual/hearing/cognitive impairment, lone workers, night workers, young persons, pregnant employees, persons unfamiliar with premises.

## 6.3 Step 3: Evaluate, Remove, Reduce, Protect

Assess adequacy of existing fire precautions across six categories:

1. Fire Detection and Warning — Type, coverage, maintenance status
2. Means of Escape — Routes, travel distances, signage, lighting
3. Emergency Lighting — Coverage, testing regime, battery backup
4. Firefighting Equipment — Type, location, suitability, maintenance
5. Signs and Notices — Fire action notices, exit signs, extinguisher signs
6. Management and Maintenance — Housekeeping, testing, training

Apply 5x5 risk matrix: Likelihood (1-5) × Consequence (1-5) = Risk Score
Risk Levels: Trivial (1-2), Low (3-4), Medium (5-9), High (10-14), Very High (15-19), Intolerable (20-25)

## 6.4 Step 4: Record, Plan, Inform, Instruct, Train

Record all significant findings in the FRA report. Prepare prioritised action plan with: finding, required action, priority (Critical/High/Medium/Low), responsible person, target date. Inform all relevant persons of the findings. Ensure emergency plan is in place. Arrange or confirm fire safety training for staff.

## 6.5 Step 5: Review

Review the FRA: at least annually, after any fire or significant near-miss, upon any material change to premises (layout, use, process, occupancy), when requested by the fire authority, after any significant building work.

Record the trigger for review and any changes made.

# 7. Building Safety Act Compliance

From October 2023, the following are mandatory: Full written record of the fire risk assessment (not just significant findings). Fire safety arrangements must be documented. Records must be shared with incoming Responsible Person on change of management/ownership.

# 8. Records Retention

FRA records shall be retained for the life of the building or a minimum of 40 years, whichever is longer. Superseded FRAs shall be archived, not destroyed.

# Appendix A: FRA Competency Criteria

The assessor shall demonstrate:

Formal fire safety qualification (IFE, NEBOSH Fire Certificate, or equivalent)
Membership of relevant professional body (IFE, IFSM) or BAFE registration
Minimum 2 years' practical fire risk assessment experience
Knowledge of current fire safety legislation and guidance
CPD record demonstrating ongoing competence

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Prepared by | [Name] |  | [DD/MM/YYYY] |
| Approved by | [Managing Director] |  | [DD/MM/YYYY] |