NEXARA

Integrated Management System

CCP Register

ISO 22000:2018 Clause 8.5.2

| Document Number: | REG-034 |
| --- | --- |
| Version: | 1.0 |
| Date: | [DD/MM/YYYY] |
| Owner: | [Food Safety Team Leader] |
| Approved By: | [Managing Director] |
| Classification: | Confidential |

Revision History

| Version | Date | Description | Author | Approved By |
| --- | --- | --- | --- | --- |
| 1.0 | [DD/MM/YYYY] | Initial Issue | [Author Name] | [Approver Name] |

| CCP # | Process Step | Hazard Controlled | Critical Limit | Monitoring Method | Frequency | Corrective Action | Verification | Records |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CCP-001 | Cooking | Survival of pathogens (B) | >=75°C core for 2 min | Probe thermometer | Every batch | Re-cook or dispose | Daily record review, thermometer cal. | FRM-042 |
| CCP-002 | Chilling | Growth of pathogens (B) | <=5°C within 4 hours | Temperature logger | Continuous | Extend chill time or dispose | Logger download, calibration | FRM-042 |
| CCP-003 | Metal detection | Metal contamination (P) | Fe 1.5mm, NFe 2.0mm, SS 2.5mm | Metal detector | All units | Reject, isolate, re-run, investigate | Test piece checks every 30 min | FRM-042 |

# 2. Review

CCP register reviewed by HACCP team at least annually and following any HACCP plan update.