NEXARA

Integrated Management System

Safe Working Procedure — Confined Space Entry

ISO 45001:2018 Clause 8.1.2

| Document Number: | SWP-002 |
| --- | --- |
| Version: | 1.0 |
| Date: | [DD/MM/YYYY] |
| Owner: | [H&S Manager] |
| Approved By: | [Operations Director] |
| Classification: | Confidential |

Revision History

| Version | Date | Description | Author | Approved By |
| --- | --- | --- | --- | --- |
| 1.0 | [DD/MM/YYYY] | Initial Issue | [Author Name] | [Approver Name] |

# 1. Purpose

To establish safe systems of work for entry into and work within confined spaces, ensuring compliance with the Confined Spaces Regulations 1997 and ISO 45001:2018.

# 2. Scope

Applies to all confined space entries including tanks, vessels, pits, silos, chambers, ducts, sewers, excavations deeper than 1.2m, and any enclosed or partially enclosed space with limited access/egress where a specified risk exists.

# 3. Definitions

- Confined Space: Any place which is substantially enclosed and where a specified risk of serious injury exists from hazardous substances, lack of oxygen, engulfment, or entrapment

- Specified Risk: Risk of fire/explosion, loss of consciousness from gas/fumes/vapour, drowning, asphyxiation from free-flowing solid, entrapment preventing escape

- Entry Supervisor: Competent person responsible for authorising and managing the confined space entry

- Entrant: Person who physically enters the confined space

- Attendant (Top Person): Person stationed outside to maintain communication and initiate emergency response

# 4. Hazards

- Toxic atmosphere (H₂S, CO, methane, solvent vapours)

- Oxygen deficiency (<19.5%) or enrichment (>23.5%)

- Flammable/explosive atmospheres

- Engulfment by liquids, slurry, or free-flowing solids

- Entrapment by converging walls or equipment

- Electrocution from equipment within the space

- Thermal hazards — extreme heat or cold

# 5. Procedure

## 5.1 Avoidance

Before any confined space entry, assess whether the work can be completed without entry (e.g., remote inspection cameras, long-handled tools, external cleaning methods). Entry is only permitted when no reasonably practicable alternative exists.

## 5.2 Pre-Entry Requirements

- Obtain a Confined Space Entry Permit (FRM-006 with confined space addendum)

- Conduct atmospheric testing: O₂ (19.5-23.5%), LEL (<10%), H₂S (<10ppm), CO (<25ppm)

- Isolate all energy sources — lock out/tag out (LOTO) all mechanical, electrical, pneumatic, hydraulic

- Isolate all pipework — blank/disconnect or double-block-and-bleed

- Purge/ventilate the space — continuous forced ventilation during entry

- Confirm rescue plan and equipment are in place before any entry

- Assign Entry Supervisor, Entrant(s), and Attendant — minimum 3 persons

## 5.3 During Entry

- Continuous atmospheric monitoring throughout the entry

- Attendant maintains constant visual/verbal contact — never leaves the entry point

- Entrant wears full-body harness with retrieval line (where practicable)

- Communication check every 5 minutes minimum

- EVACUATE IMMEDIATELY if: atmosphere alarm sounds, ventilation fails, entrant feels unwell, any emergency

## 5.4 Emergency Rescue

NEVER enter a confined space to rescue a casualty without breathing apparatus and a trained rescue team.

a) Attendant raises alarm immediately — calls emergency services (999)
b) Attempt non-entry rescue first (retrieval line/winch)
c) Entry rescue only by trained rescue team wearing SCBA
d) Administer first aid once the casualty is removed to fresh air
e) All casualties to be assessed at hospital even if apparently uninjured

| Equipment | Purpose | Check |
| --- | --- | --- |
| 4-gas monitor | Continuous atmospheric monitoring | Calibrated within 6 months, bump tested |
| Forced ventilation fan | Maintain breathable atmosphere | Operational check, correct ducting |
| Full-body harness + tripod/winch | Fall prevention + rescue retrieval | Inspected, rated for entrant weight |
| SCBA sets (x2 minimum) | Emergency rescue entry | Full cylinders, face-fit tested |
| Communication equipment | Entrant-attendant contact | Tested before entry |
| Intrinsically safe lighting | Illumination in space | IS rated, battery charged |

# 7. Training

- Confined Space Awareness — all relevant employees

- Confined Space Entrant — practical entry procedures

- Entry Supervisor — permit management, atmospheric monitoring

- Confined Space Rescue — rescue team (annual refresher)

- Gas Monitor Use — all entrants and attendants

# 8. Related Documents

- FRM-006: Permit to Work Form

- PRO-005: HIRA Procedure

- Confined Spaces Regulations 1997

- Approved Code of Practice L101