NEXARA

Integrated Management System

Safe Working Procedure — Manual Handling

ISO 45001:2018 Clause 8.1.2

| Document Number: | SWP-005 |
| --- | --- |
| Version: | 1.0 |
| Date: | [DD/MM/YYYY] |
| Owner: | [H&S Manager] |
| Approved By: | [Operations Director] |
| Classification: | Confidential |

Revision History

| Version | Date | Description | Author | Approved By |
| --- | --- | --- | --- | --- |
| 1.0 | [DD/MM/YYYY] | Initial Issue | [Author Name] | [Approver Name] |

# 1. Purpose

To reduce the risk of musculoskeletal disorders (MSDs) from manual handling operations, in compliance with the Manual Handling Operations Regulations 1992 (as amended) and ISO 45001:2018.

# 2. Scope

Applies to all manual handling operations: lifting, lowering, pushing, pulling, carrying, holding, and restraining any load (including persons). Covers all employees and contractors.

| Factor | Considerations | Control Examples |
| --- | --- | --- |
| Task | Distance carried, height of lift, frequency, duration, twisting, stooping | Reduce carry distance, avoid lifting from floor level, rotate tasks |
| Individual | Fitness, strength, existing injuries, pregnancy, training | Health surveillance, task rotation, training in techniques |
| Load | Weight, size, shape, grip, stability, temperature | Reduce load weight, provide handles, break into smaller loads |
| Environment | Space, floor surface, lighting, temperature, slopes | Improve housekeeping, ensure adequate lighting, level surfaces |

| Height Zone | Close to Body (M/F) | Arms Extended (M/F) |
| --- | --- | --- |
| Shoulder height | 10kg / 7kg | 5kg / 3kg |
| Elbow height | 20kg / 13kg | 10kg / 7kg |
| Knuckle height | 25kg / 16kg | 15kg / 10kg |
| Mid lower leg | 10kg / 7kg | 5kg / 3kg |

# 5. Safe Lifting Technique

- Plan the lift — assess the load, route, and destination

- Position feet hip-width apart with one foot slightly forward

- Bend knees, keep back straight, grip the load firmly

- Lift smoothly using leg muscles — avoid jerking

- Keep the load close to the body, arms below shoulder height

- Avoid twisting — turn using feet, not the spine

- Set down by reversing the procedure — bend knees, controlled lowering

# 6. Mechanical Aids

Where manual handling cannot be avoided, provide mechanical aids:

- Pallet trucks and trolleys for horizontal movement

- Hoists and cranes for heavy/awkward loads

- Vacuum lifters for sheet materials

- Conveyor systems for repetitive movements

- Height-adjustable workstations to reduce stooping/reaching

# 7. Training

All employees involved in manual handling shall receive:

a) Manual Handling Awareness training on induction
b) Task-specific manual handling assessment for their role
c) Refresher training every 3 years or following any MSD report
d) Practical sessions covering correct lifting techniques

# 8. Related Documents

- FRM-002: Hazard & Risk Assessment Form

- Manual Handling Operations Regulations 1992

- HSE INDG143: Manual Handling at Work

- Occupational Health Referral Process